CPE /EE 556 A: Assignment two

Matthew Marseglia 3/10/26

Reading: Sections 15.7 and 15.8

Instructions:

- upload one Microsoft text document that includes commented code for each part and YouTube links to the video demos.

- This assignment is out of 12 points

- Important Note: Please do not use any of the revised pins. Otherwise, your board will stop working.

Part 1: The Simple Blink

Q1. Modify the ARM code to blink the LED at a slower speed.

Q2. Modify the ARM code to blink an out-of-board LED.

Video submission for Q1 and Q2: https://youtube.com/shorts/qguE8aObYsA

Q1 and Q2: Changed r2 = 0x25000 to r2 = 0x500000

Part 2: Button Blink

Q3. Modify the ARM code such that it sets a Pin as an output and another Pin as an input. If the button is pressed the LED will be Toggled. Replace the (AND+CMP) instructions with TST and. Please do not use the on-board button or LED.

Q3Videosubmission: https://youtube.com/shorts/5kg_3cJR8AM?feature=share

progloop ;program runs in an infinite loop

LDR r0, =GPIOC_BASE

LDR r1, [r0, #GPIO_IDR]

TST r1,  #(1<<USR_BUTTON)

;AND r2, r1,  #(1<<USR_BUTTON) 	; select bit 13 in IDR register for USR_BUTTON

;CMP r2, #(1<<USR_BUTTON) 	; check if button is pressed

BNE progloop 		; restart loop if button is not pressed

Part 3: Multiple Blink

Q4: Now that we have blinked at least 1 LED, what about blinking multiple LEDs at the same time? The minimum that you need to develop is blinking at least three LEDs at different rates. Although I am not going to give you speed, you should probably pick a rate, which is visible to a standard human.

Q4 Video submission: https://youtube.com/shorts/43Ap_DWxeCE

Updated Code:

; Green LED <--> PA.5

RED_PIN	EQU	5

YELLOW_PIN EQU 6

BLUE_PIN EQU 7

GPIO_MODER      EQU  0x00      ; GPIO port mode register,               Address offset: 0x00

GPIO_OTYPER     EQU  0x04      ; GPIO port output type register,        Address offset: 0x04

GPIO_OSPEEDR    EQU  0x08      ; GPIO port output speed register,       Address offset: 0x08

GPIO_PUPDR      EQU  0x0C      ; GPIO port pull-up/pull-down register,  Address offset: 0x0C

GPIO_IDR        EQU  0x10      ; GPIO port input data register,         Address offset: 0x10

GPIO_ODR        EQU  0x14      ; GPIO port output data register,        Address offset: 0x14

GPIO_BSRR       EQU  0x18      ; GPIO port bit set/reset  register,     Address offset: 0x18

GPIO_LCKR       EQU  0x1C      ; GPIO port configuration lock register, Address offset: 0x1C

GPIO_AFR0       EQU  0x20      ; GPIO alternate function registers,     Address offset: 0x20-0x24

GPIO_AFR1       EQU  0x24      ; GPIO alternate function registers,     Address offset: 0x20-0x24

GPIO_BRR        EQU  0x28      ; GPIO Bit Reset register,               Address offset: 0x28

GPIO_ASCR       EQU  0x2C      ; GPIO analog switch control register,   Address offset: 0x2C

PERIPH_BASE   EQU (0x40000000) ; Peripheral base address

AHB2PERIPH_BASE       EQU (PERIPH_BASE + 0x08000000); 0x4800 0000

GPIOA_BASE    EQU (AHB2PERIPH_BASE + 0x0000); 0x4800 0000

AHB1PERIPH_BASE       EQU (PERIPH_BASE + 0x00020000); 0x4002 0000

RCC_BASE      EQU (AHB1PERIPH_BASE + 0x1000)	; 0x4002 1000

RCC_AHB2ENR     EQU  0x4C  ; RCC AHB2 peripheral clock enable register, Address offset: 0x4C

RCC_AHB2ENR_GPIOAEN         EQU (0x00000001)

AREA    main, CODE, READONLY

EXPORT	__main				; make __main visible to linker

ENTRY

__main	PROC

; Enable the clock to GPIO Port A

LDR r0, =RCC_BASE

LDR r1, [r0, #RCC_AHB2ENR]

ORR r1, r1, #RCC_AHB2ENR_GPIOAEN

STR r1, [r0, #RCC_AHB2ENR]

; MODE: 00: Input mode, 01: General purpose output mode

;       10: Alternate function mode, 11: Analog mode (reset state)

LDR r0, =GPIOA_BASE

LDR r1, [r0, #GPIO_MODER]

BIC r1, r1, #(3<<(2*RED_PIN))

ORR r1, r1, #(1<<(2*RED_PIN))    ;output (01)

BIC r1, r1, #(3<<(2*YELLOW_PIN))

ORR r1, r1, #(1<<(2*YELLOW_PIN))    ;output (01)

BIC r1, r1, #(3<<(2*BLUE_PIN))

ORR r1, r1, #(1<<(2*BLUE_PIN))    ;output (01)

STR r1, [r0, #GPIO_MODER]

; OUTPUT TYPE: 0: Push-Pull, 1: Open drain

LDR r1, [r0, #GPIO_OTYPER]

AND r1, r1,  #~(1<<RED_PIN)

AND r1, r1,  #~(1<<YELLOW_PIN)

AND r1, r1,  #~(1<<BLUE_PIN)

STR r1, [r0, #GPIO_OTYPER]

LDR r2, = 0x500000

LDR r3, = 0x500000

LDR r4, = 0x500000

Repeat		SUBS r4, r4, #4

SUBS r3, r3, #2

SUBS r2, r2, #1

redCheck	CMP r2, #0

BNE yellowCheck

LDR r1, [r0, #GPIO_ODR]

EOR r1, r1, #(1<<RED_PIN)        ;toggle LED

STR r1, [r0, #GPIO_ODR]

LDR r2, = 0x500000		 		;reset the timer

yellowCheck	CMP r3, #0

BNE blueCheck

LDR r1, [r0, #GPIO_ODR]

EOR r1, r1, #(1<<YELLOW_PIN)        ;toggle LED

STR r1, [r0, #GPIO_ODR]

LDR r3, = 0x500000		 		;reset the timer

blueCheck	CMP r4, #0

BNE Repeat

LDR r1, [r0, #GPIO_ODR]

EOR r1, r1, #(1<<BLUE_PIN)        ;toggle LED

STR r1, [r0, #GPIO_ODR]

LDR r4, = 0x500000		 		;reset the timer

B 		Repeat  		; dead loop & program hangs here

;ENDP

END

Part 4: Use the 1 discovery kit to do the following

Q5: Use C programing to blink the on-board user LEDS: LD6 and LD7. The C code should pool the on-board user button (B3), if the button is clicked the LEDs blink, otherwise they are OFF.

datasheet of the B-U585I-IOT02A Discovery kit for the IoT node

Q5Videosubmission: https://youtube.com/shorts/4FOd5BXFaO8?feature=share

#include "stm32u585xx.h"

#define LD6     6

#define LD7     7

#define BUTTON  13

void LEDInit(void) {

// 1. Enable Clock for GPIOH

RCC->AHB2ENR1 |= RCC_AHB2ENR1_GPIOHEN;

// 2. Configure PH6 and PH7 as Output (01)

GPIOH->MODER &= ~((3UL << (2 * LD6)) | (3UL << (2 * LD7)));

GPIOH->MODER |=  ((1UL << (2 * LD6)) | (1UL << (2 * LD7)));

// 3. Set to Medium Speed (01)

GPIOH->OSPEEDR &= ~((3UL << (2 * LD6)) | (3UL << (2 * LD7)));

GPIOH->OSPEEDR |=  ((1UL << (2 * LD6)) | (1UL << (2 * LD7)));

// 4. Set to Push-Pull (Reset bits to 0)

GPIOH->OTYPER &= ~((1UL << LD6) | (1UL << LD7));

}

void buttonInit(void) {

// Enable Clock for GPIOC

RCC->AHB2ENR1 |= RCC_AHB2ENR1_GPIOCEN;

// Set PC13 to Input mode (00)

GPIOC->MODER &= ~(3UL << (2 * BUTTON));

// Set PC13 to Pull-Down (10)

GPIOC->PUPDR &= ~(3UL << (2 * BUTTON));

GPIOC->PUPDR |=  (2UL << (2 * BUTTON));

}

int main(void) {

LEDInit();

buttonInit();

while(1) {

// Poll the button: If High (pressed)

if (GPIOC->IDR & (1UL << BUTTON)) {

// Toggle LEDs

GPIOH->ODR ^= ((1UL << LD6) | (1UL << LD7));

// Blink Delay - Use 'volatile' so the compiler doesn't optimize it away

for(volatile int i = 0; i < 100000; i++);

}

else {

// Requirement: "otherwise they are OFF"

GPIOH->ODR &= ~((1UL << LD6) | (1UL << LD7));

}

}

}

Part 5: generative AI

Q6: Explain how do generative AI tools like ChatGPT or Copilot have helped you with the assignment

Generative AI tools such as ChatGPT, Gemini, and Copilot have helped with this assignment by providing further analysis of the code depth to intuitively understand the microcontroller processes.  Generative AI also is helpful with follow up questions about the code that the assignment itself does not specify.  This allowed for further analysis beyond the scope of this assignment.  Generative AI also helps us to understand different arguments within the code if there are no comments provided to explain certain steps that are not clear to the user.  AI was helpful was setting up header files as well.