CPE /EE 556 A: Assignment four

# Reading:

- Read sections 16.1-16.3

# Instructions:

- upload one Microsoft text document that includes commented code for each part and YouTube links to the video demos.

- This assignment is out of 12 points.

- Please follow the example video demo posted on Canvas.

- Important Note: Please do not use any of the revised pins. Otherwise, your board will stop working.

# Using timer to toggle 4 LEDs.

- Create a project that toggles four external LEDs at the following frequencies: 0.5 Hz, 1 Hz, 2 Hz, and 4 Hz.

- Compute the necessary timer clock frequency (fCL_CNT) and the CCR values required to achieve these toggling rates.

- The default system clock is 4 MHz.

Video submission: https://youtu.be/69pegLJwlFE

#include "stm32l476xx.h"

// 0.5Hz = TIM2_CH2 = PA5 = A1  (AF1)

// 1Hz   = TIM3_CH1 = PA6 = D12 (AF2)

// 2Hz   = TIM3_CH2 = PA8 = D11 (AF2)

// 4Hz   = TIM4_CH1 = PB6 = D10 (AF2)

void LED_Pin_Init() {

// GPIOA: PA1, PA6, PA7

RCC->AHB2ENR  |= RCC_AHB2ENR_GPIOAEN;

// PA1 - TIM2_CH2 - AF1

GPIOA->MODER  &= ~(0x03 << (2*1));

GPIOA->MODER  |=   0x02 << (2*1);

GPIOA->AFR[0] &= ~(0xF  << (4*1));

GPIOA->AFR[0] |=   0x1  << (4*1);      // AF1 = TIM2

GPIOA->OTYPER &= ~(1<<1);

// PA6 - TIM3_CH1 - AF2

GPIOA->MODER  &= ~(0x03 << (2*6));

GPIOA->MODER  |=   0x02 << (2*6);

GPIOA->AFR[0] &= ~(0xF  << (4*6));

GPIOA->AFR[0] |=   0x2  << (4*6);      // AF2 = TIM3

GPIOA->OTYPER &= ~(1<<6);

// PA7 - TIM3_CH2 - AF2

GPIOA->MODER  &= ~(0x03 << (2*7));

GPIOA->MODER  |=   0x02 << (2*7);

GPIOA->AFR[0] &= ~(0xF  << (4*7));

GPIOA->AFR[0] |=   0x2  << (4*7);      // AF2 = TIM3

GPIOA->OTYPER &= ~(1<<7);

// GPIOB: PB6 - TIM4_CH1 - AF2

RCC->AHB2ENR  |= RCC_AHB2ENR_GPIOBEN;

GPIOB->MODER  &= ~(0x03 << (2*6));

GPIOB->MODER  |=   0x02 << (2*6);

GPIOB->AFR[0] &= ~(0xF  << (4*6));

GPIOB->AFR[0] |=   0x2  << (4*6);      // AF2 = TIM4

GPIOB->OTYPER &= ~(1<<6);

}

void TIM2_CH2_Init() {  // PA1 - 0.5 Hz

RCC->APB1ENR1 |= RCC_APB1ENR1_TIM2EN;

TIM2->CR1  &= ~TIM_CR1_DIR;

TIM2->PSC   =  39999;

TIM2->ARR   =  199;

TIM2->CCMR1 &= ~TIM_CCMR1_OC2M;

TIM2->CCMR1 |=  TIM_CCMR1_OC2M_0 | TIM_CCMR1_OC2M_1;  // Toggle

TIM2->CCR2   =  0;

TIM2->CCER  |=  TIM_CCER_CC2E;

TIM2->EGR   |=  TIM_EGR_UG;

TIM2->SR    &= ~TIM_SR_UIF;

TIM2->CR1   |=  TIM_CR1_CEN;

}

void TIM3_CH12_Init() {  // PA6 - 1Hz, PA7 - 2Hz

RCC->APB1ENR1 |= RCC_APB1ENR1_TIM3EN;

TIM3->CR1  &= ~TIM_CR1_DIR;

TIM3->PSC   =  39999;

TIM3->ARR   =  99;           // 1 Hz period

// CH1: PA6 - 1 Hz

TIM3->CCMR1 &= ~TIM_CCMR1_OC1M;

TIM3->CCMR1 |=  TIM_CCMR1_OC1M_0 | TIM_CCMR1_OC1M_1;  // Toggle

TIM3->CCR1   =  0;

TIM3->CCER  |=  TIM_CCER_CC1E;

// CH2: PA7 - 2 Hz (CCR2 at midpoint so it toggles twice per ARR cycle)

TIM3->CCMR1 &= ~TIM_CCMR1_OC2M;

TIM3->CCMR1 |=  TIM_CCMR1_OC2M_0 | TIM_CCMR1_OC2M_1;  // Toggle

TIM3->CCR2   =  49;

TIM3->CCER  |=  TIM_CCER_CC2E;

TIM3->EGR   |=  TIM_EGR_UG;

TIM3->SR    &= ~TIM_SR_UIF;

TIM3->CR1   |=  TIM_CR1_CEN;

}

void TIM4_CH1_Init() {  // PB6 - 4 Hz

RCC->APB1ENR1 |= RCC_APB1ENR1_TIM4EN;

TIM4->CR1  &= ~TIM_CR1_DIR;

TIM4->PSC   =  39999;

TIM4->ARR   =  24;

TIM4->CCMR1 &= ~TIM_CCMR1_OC1M;

TIM4->CCMR1 |=  TIM_CCMR1_OC1M_0 | TIM_CCMR1_OC1M_1;  // Toggle

TIM4->CCR1   =  0;

TIM4->CCER  |=  TIM_CCER_CC1E;

TIM4->EGR   |=  TIM_EGR_UG;

TIM4->SR    &= ~TIM_SR_UIF;

TIM4->CR1   |=  TIM_CR1_CEN;

}

int main(void) {

LED_Pin_Init();

TIM2_CH2_Init();

TIM3_CH12_Init();

TIM4_CH1_Init();

while(1);

}

# Using timer to toggle the onboard LED of the B-U585I-IOT02A Discovery kit at a rate of 0.5 Hz.

- The onboard LEDs are not connected to any timer channel.

- Use a general-purpose timer to create the delay between the ON and OFF.

- When the timer overflows/underfollows, it calls an interrupt.

- Toggle the LED inside the interrupt service routine.

Video submission: https://youtu.be/rD3yp37Mu-k

#include "stm32u585xx.h"

// LD1 (green) = PH7

// TIM3 overflow at 1 Hz ? toggle every overflow ? 0.5 Hz blink

void LED_Init(void) {

RCC->AHB2ENR1  |= RCC_AHB2ENR1_GPIOHEN;   // U5: AHB2ENR1, not AHB2ENR

GPIOH->MODER   &= ~(0x03 << (2*7));

GPIOH->MODER   |=  (0x01 << (2*7));        // output mode

GPIOH->OTYPER  &= ~(1<<7);                 // push-pull

GPIOH->OSPEEDR &= ~(0x03 << (2*7));        // low speed

GPIOH->PUPDR   &= ~(0x03 << (2*7));        // no pull

GPIOH->ODR     &= ~(1<<7);                 // start OFF

}

void TIM3_Init(void) {

// 4 MHz MSIS / (PSC+1=4000) / (ARR+1=1000) = 1 Hz interrupt

RCC->APB1ENR1 |= RCC_APB1ENR1_TIM3EN;

TIM3->CR1  &= ~TIM_CR1_DIR;    // upcounting

TIM3->PSC   =  3999;

TIM3->ARR   =  999;

TIM3->DIER |=  TIM_DIER_UIE;   // enable update interrupt

TIM3->EGR  |=  TIM_EGR_UG;     // force register update

TIM3->SR   &= ~TIM_SR_UIF;     // clear any pending flag

NVIC_SetPriority(TIM3_IRQn, 1);

NVIC_EnableIRQ(TIM3_IRQn);

TIM3->CR1  |=  TIM_CR1_CEN;    // start timer

}

// ISR name must match exactly — no parameters, no return type

void TIM3_IRQHandler(void) {

if (TIM3->SR & TIM_SR_UIF) {

TIM3->SR &= ~TIM_SR_UIF;   // clear flag first, then act

GPIOH->ODR ^= (1 << 7);    // toggle LD1

}

}

int main(void) {

LED_Init();

TIM3_Init();

while (1);

}

# Using a timer to generate 5 PWM signals

- Create a project that generates five PWM signals with duty cycles of 10%, 20%, 30%, 50%, and 60%.

- These signals should control the intensity of five external LEDs.

- Select an appropriate PWM frequency and explain your choice.

Video submission: https://youtube.com/shorts/YUoIZKBf2co

#include "stm32l476xx.h"

// PWM frequency = 4MHz / (40 * 1000) = 100 Hz

// ARR = 999, CCR = duty% * 10

// TIM2_CH1=PA0=A0(10%), TIM2_CH2=PA1=A1(20%), TIM3_CH1=PA6=D12(30%)

// TIM3_CH2=PA7=D11(50%), TIM4_CH1=PB6=D10(60%)

void LED_Pin_Init() {

// GPIOA: PA0, PA1, PA6, PA7

RCC->AHB2ENR  |= RCC_AHB2ENR_GPIOAEN;

GPIOA->MODER  &= ~(0x03<<(2*0) | 0x03<<(2*1) | 0x03<<(2*6) | 0x03<<(2*7));

GPIOA->MODER  |=  (0x02<<(2*0) | 0x02<<(2*1) | 0x02<<(2*6) | 0x02<<(2*7));

// PA0=AF1(TIM2), PA1=AF1(TIM2), PA6=AF2(TIM3), PA7=AF2(TIM3)

GPIOA->AFR[0] &= ~(0xF<<(4*0) | 0xF<<(4*1) | 0xF<<(4*6) | 0xF<<(4*7));

GPIOA->AFR[0] |=  (0x1<<(4*0) | 0x1<<(4*1) | 0x2<<(4*6) | 0x2<<(4*7));

GPIOA->OTYPER &= ~(1<<0 | 1<<1 | 1<<6 | 1<<7);

// GPIOB: PB6

RCC->AHB2ENR  |= RCC_AHB2ENR_GPIOBEN;

GPIOB->MODER  &= ~(0x03<<(2*6));

GPIOB->MODER  |=  (0x02<<(2*6));

// PB6=AF2(TIM4)

GPIOB->AFR[0] &= ~(0xF<<(4*6));

GPIOB->AFR[0] |=  (0x2<<(4*6));

GPIOB->OTYPER &= ~(1<<6);

}

void TIM2_PWM_Init() {

RCC->APB1ENR1 |= RCC_APB1ENR1_TIM2EN;

TIM2->CR1     &= ~TIM_CR1_DIR;

TIM2->PSC      =  39;

TIM2->ARR      =  999;

// CH1: PA0 - 10%

TIM2->CCMR1 &= ~TIM_CCMR1_OC1M;

TIM2->CCMR1 |=  TIM_CCMR1_OC1M_2 | TIM_CCMR1_OC1M_1;

TIM2->CCR1   =  100;

TIM2->CCER  |=  TIM_CCER_CC1E;

// CH2: PA1 - 20%

TIM2->CCMR1 &= ~TIM_CCMR1_OC2M;

TIM2->CCMR1 |=  TIM_CCMR1_OC2M_2 | TIM_CCMR1_OC2M_1;

TIM2->CCR2   =  200;

TIM2->CCER  |=  TIM_CCER_CC2E;

TIM2->EGR   |=  TIM_EGR_UG;

TIM2->SR    &= ~TIM_SR_UIF;

TIM2->CR1   |=  TIM_CR1_CEN;

}

void TIM3_PWM_Init() {

RCC->APB1ENR1 |= RCC_APB1ENR1_TIM3EN;

TIM3->CR1     &= ~TIM_CR1_DIR;

TIM3->PSC      =  39;

TIM3->ARR      =  999;

// CH1: PA6 - 30%

TIM3->CCMR1 &= ~TIM_CCMR1_OC1M;

TIM3->CCMR1 |=  TIM_CCMR1_OC1M_2 | TIM_CCMR1_OC1M_1;

TIM3->CCR1   =  300;

TIM3->CCER  |=  TIM_CCER_CC1E;

// CH2: PA7 - 50%

TIM3->CCMR1 &= ~TIM_CCMR1_OC2M;

TIM3->CCMR1 |=  TIM_CCMR1_OC2M_2 | TIM_CCMR1_OC2M_1;

TIM3->CCR2   =  500;

TIM3->CCER  |=  TIM_CCER_CC2E;

TIM3->EGR   |=  TIM_EGR_UG;

TIM3->SR    &= ~TIM_SR_UIF;

TIM3->CR1   |=  TIM_CR1_CEN;

}

void TIM4_PWM_Init() {

RCC->APB1ENR1 |= RCC_APB1ENR1_TIM4EN;

TIM4->CR1     &= ~TIM_CR1_DIR;

TIM4->PSC      =  39;

TIM4->ARR      =  999;

// CH1: PB6 - 60%

TIM4->CCMR1 &= ~TIM_CCMR1_OC1M;

TIM4->CCMR1 |=  TIM_CCMR1_OC1M_2 | TIM_CCMR1_OC1M_1;

TIM4->CCR1   =  600;

TIM4->CCER  |=  TIM_CCER_CC1E;

TIM4->EGR   |=  TIM_EGR_UG;

TIM4->SR    &= ~TIM_SR_UIF;

TIM4->CR1   |=  TIM_CR1_CEN;

}

int main(void) {

LED_Pin_Init();

TIM2_PWM_Init();

TIM3_PWM_Init();

TIM4_PWM_Init();

while(1);}

# Using timer to generate PWM signal with variable duty cycle

- Set up a timer to generate a PWM signal where the duty cycle varies from 10% to 100% in steps of 10%.

- The duty cycle should increase with each press of a pushbutton and restart at 10% after reaching 100%.

Video submission: https://youtu.be/SkFSVOBwPLo

#include "stm32l476xx.h"

// PWM frequency = 4MHz / (40 * 1000) = 100 Hz

// ARR = 999, duty step = 100 (10%)

// TIM2_CH2 = PA1 = A1 (AF1)

// Button = PC13 = onboard blue button, idle=HIGH, pressed=LOW

int duty = 100;

void LED_Pin_Init() {

RCC->AHB2ENR  |= RCC_AHB2ENR_GPIOAEN;

GPIOA->MODER  &= ~(0x03<<(2*1));

GPIOA->MODER  |=  (0x02<<(2*1));

GPIOA->AFR[0] &= ~(0xF<<(4*1));

GPIOA->AFR[0] |=  (0x1<<(4*1));         // AF1 = TIM2_CH2

GPIOA->OTYPER &= ~(1<<1);

}

void Button_Init() {

RCC->AHB2ENR  |= RCC_AHB2ENR_GPIOCEN;

GPIOC->MODER  &= ~(0x03<<(2*13));       // Input mode

GPIOC->PUPDR  &= ~(0x03<<(2*13));       // No pull, board has external pull-up

}

void TIM2_PWM_Init() {

RCC->APB1ENR1 |= RCC_APB1ENR1_TIM2EN;

TIM2->CR1     &= ~TIM_CR1_DIR;

TIM2->PSC      =  39;

TIM2->ARR      =  999;

TIM2->CCMR1   &= ~TIM_CCMR1_OC2M;

TIM2->CCMR1   |=  TIM_CCMR1_OC2M_2 | TIM_CCMR1_OC2M_1;

TIM2->CCR2     =  duty;

TIM2->CCER    |=  TIM_CCER_CC2E;

TIM2->EGR     |=  TIM_EGR_UG;

TIM2->SR      &= ~TIM_SR_UIF;

TIM2->CR1     |=  TIM_CR1_CEN;

}

int main(void) {

LED_Pin_Init();

Button_Init();

TIM2_PWM_Init();

while(1) {

if (!((GPIOC->IDR >> 13) & 1)) {    // button pressed (LOW)

duty += 100;

if (duty > 999) duty = 100;

TIM2->CCR2 = duty;

while (!((GPIOC->IDR >> 13) & 1));  // wait for release

}

}

}

# Using a timer to generate PWM signal at one of the GPIO pins of the B-U585I-IOT02A Discovery kit

- Create a project that generates a PWM signal with a duty cycle of 35%.

- Select an appropriate PWM frequency and explain your choice.

Video submission: https://youtu.be/TdqUw9I2L8c

#include "stm32u585xx.h"

#define PWM_FREQ    5000       // 5 kHz

#define TIMER_CLOCK 100000000  // 100 MHz timer clock

int main(void)

{

// 1. Enable GPIOA clock

RCC->AHB2ENR1 |= RCC_AHB2ENR1_GPIOAEN;

// 2. Configure PA8 as alternate function TIM1_CH1

GPIOA->MODER &= ~(0x3 << (8 * 2));    // Clear mode

GPIOA->MODER |=  (0x2 << (8 * 2));    // Set AF mode

GPIOA->AFR[1] &= ~(0xF << ((8-8)*4)); // Clear AF for PA8

GPIOA->AFR[1] |=  (0x2 << ((8-8)*4)); // AF2 = TIM1_CH1

// 3. Enable TIM1 clock

RCC->APB2ENR |= RCC_APB2ENR_TIM1EN;

// 4. Set prescaler and auto-reload

TIM1->PSC = 0;  // No prescaler

TIM1->ARR = (TIMER_CLOCK / PWM_FREQ) - 1; // Auto-reload value

// 5. Set compare value for 35% duty cycle

TIM1->CCR1 = ((TIM1->ARR + 1) * 35) / 100;

// 6. Configure PWM mode 1 on channel 1

TIM1->CCMR1 &= ~TIM_CCMR1_OC1M;          // Clear mode

TIM1->CCMR1 |= (0x6 << TIM_CCMR1_OC1M_Pos); // PWM mode 1

TIM1->CCMR1 |= TIM_CCMR1_OC1PE;          // Enable preload

// 7. Enable output on channel 1

TIM1->CCER |= TIM_CCER_CC1E;

// 8. Enable main output (required for advanced timers like TIM1)

TIM1->BDTR |= TIM_BDTR_MOE;

// 9. Enable auto-reload preload

TIM1->CR1 |= TIM_CR1_ARPE;

// 10. Start the timer

TIM1->CR1 |= TIM_CR1_CEN;

while(1)

{

// PWM runs automatically on PA8 (CN13 D9)

}

}